Operating Instructions

1x2 HDMI Splitter Over Single Cat5e/6 Cable

Up to 50M with IR

Model: H2SPC5L

Operating Instructions

Notice

The information in this document is subject to change without notice. Every effort has been made in the preparation of this document to ensure accuracy of the contents, but all statements, information, and recommendations in this document do not constitute the warranty of any kind, express or implied.

Features

- Distribute 1 HDMI singal to 2 identical UTP outputs synchronously,

with 1xlooping HDMI output.

- Transmit over 50m single Cat5e/6 cable.

- Can be cascaded for 4 layers.

- With wide-band Bi-Direction IR control (38 ~ 56 kHz).

(IR extension from Sender to Receiver or from Receiver to Sender)

- Support Full HD 1080p, VESA resolution: 1920x1200.

- Supports Dolby  TrueHD, DTS-HD  Master Audio.

- Support 3D.

Package Contents

- 1 x HDMI Splitter

- 1 x 5V DC Adapter

- 1 x User Manual

1

Operating Instructions

SPECIFICATIONS

| Operating Temperature Range | -5 to +35℃(+23 to +95℉) |
| --- | --- |
| Operating Humidity Range | 5 to 90%RH (No Condensation) |
| Video Amplifier Bandwidth | 140MHz |
| Input Video Signal | 0.5-1.0 Volts P-P |
| Input DDC Signal | 5 volts p-p(TTL) |
| Data transfer speed rate | 6.75Gbps(Maximum) |
| Input port | 1xHDMI |
| Output ports | 2xCat5e/6, 1xHDMI |
| Video Format Supported | DTV/HDTV:480i/576i/480P/576P/720P/1080i/1080P VESA resolution:1920x1200① 3D video support |
| Vertical Frequency range | 60Hz |
| Support Audio Format | DTS-HD Master Audio, Dolby True HD, multi-channel 5.1, stereo etc ② |
| Transmission Distance | 1080P 8-bit 50m Over single cat5e or cat6 cable/24AWG/Solid③ |
| Power consumption | 4W(Max) |
| Dimension (L×W×H) | L149xW79xH28mm |
| Net Weight | 317.7g |

Note: ① The transmission distance is Max 40m under 1920x1200 resolution.

② The distance is 40m under 5.1CH or 7.1CH.

③ Under perfect transmission conditions including straight cable runs with no electrical interference, bends, kinks, patch panels or wall outlets.

PANEL DESCRIPTIONS

Front Panel

1. DIP switch for EDID setting            2. IR-TX port

3. IR-RX port                           4. Indicator of HDMI output

5. Indicator of HDMI input                6. Power input indicator

2

Operating Instructions

Rear Panel

7. CAT-5E/6 out ports                   8. HDMI output

9. HDMI input                         10. Power input

DIP Switch for EDID Setting

| DIP Switch | EDID |
| --- | --- |
| 0000 | Default (1080P2D 2ch) |
| 0001 | Copy from HDMI output |
| 0010 | 1080P2D 2CH |
| 0011 | 1080P2D 6CH |
| 0100 | 1080P2D 8CH |
| 0101 | Reserved |
| 0110 | 1080P3D 2CH |
| 0111 | 1080P3D 6CH |
| 1111 | 1080P3D 8CH |

Connecting and operating

1) Connect the HDMI input source into HDMI splitter (by HDMI cable).

2) Connect by single cat5e/6 cable between the Cat5e/6 outputs of HDMI splitter and Cat5e/6 input of receiver (The cable termination must follow the standard of EIA/TIA

568B).

3) Connect the HDMI display (such as HD-LCD/HD-DLP) into the HDMI output of

receiver.

4) Insert the DC 5V power supply into HDMI splitter and receiver.

3

Operating Instructions

Connection Diagram

Application 1:

Application 2: Cascade for 4 layers

4

Operating Instructions