阿INSTRUCTION MANUAL

USER MANUAL OF WiFi SOCKET

MODEL NO.: 8WD-IWSS001

Afterles Support - Help Desk

Contents

General Safety Instructions.....................3

Product Features.....................................4

Parts List / Technical Specifications.........5

Product Setup..........................................6

Product Operation...................................15

Timers & Schedules................................17

FAQ’S ....................................................25

General Safety Instructions

Read this manual thoroughly before first use and keep it in a safe place for

future reference, along with the purchase receipt and carton.

The safety precautions enclosed herein reduce the risk of fire, electric shock

and injury when correctly adhered to. Follow all instructions and take notice

of all warnings to reduce the risk of fire or electric shock.

•  Indoor use only

•  Do not exceed the maximum power rating 10A or 2400W

•  Do not expose to water, dust, chemicals and moisture

•  Keep away from heat sources and direct sunlight

•  Children and infirm persons should use this device only under the

supervision of a responsible adult to control appliances safely

•  Young children must be supervised to ensure they do not play with

this device.

•  Do not operate or control heat producing devices connected to the

Wifi socket when unattended.

•  Do not drop or subject the device to undue shock

•  Unplug this device during lightning storms or when unused for long

periods of time

Product Features

LED indicator/

Operating button  

- Control the Wifi Socket from anywhere, using your iOS/ Android Device

- Check the current status of your Wifi Socket from anywhere, using your iOS / Android device

- Easy to setup on your existing wireless network

- Set timers to activate your according to a schedule

- Control up to 100 Wifi Sockets using one iOS / Android device

- Gives you extra control to conserve energy

* Wifi Socket requires a Wi-Fi network. Mobile remote control

requires an iOS or Android device with Wi-Fi, 3G or 4G connection.

After Sales Support - Help Desk

Parts List

•  Wifi Socket

•  Getting Started Guide

•  Instruction Manual

Technical Specifications

Voltage: 240V AC 50HZ

Max load current: 10A

Total load must not exceed 2400W

Working temperature: 0-50°C

Network: 802.11, b/g/n

Security: WPA-PSK / WPA2-PSK / WPA / WPA2 / WEP / WPS2 / WAPI

Encryption Type: WEP / TKIP / AES

Product Setup

Before you can use your Wifi Socket, it must be set  up using your

Android/iOS device.

Initial setup must be performed using your home Wi-Fi network.

Once it has been set up you can control your power outlet via Wi-Fi/3G/4G.

Important: A current mobile service plan is required for correct

operation over 3G & 4G. Mobile data may be required for some

features. The supplier is not responsible for any chargers incurred.

Please contact your network service provider for further information

about your current data plan and any associated charges.

Note: Consult your device’s instruction manual if you are unsure

how to download apps, connect to Wi-Fi or any other functions of

your Android or iOS device.

Software

The free SmartPoint App is available for Android and iPhone, and is required

to operate your Wifi Socket.

1.   Ensure your iOS / Android device is connected to the internet using your

home Wi-Fi network.

2.   Open the App store/Google Market  on your phone.

3.   Search for “SmartPoint” in the App store/ Google Market

4.   Download the “SmartPoint” App to your phone.

Connecting the Wifi Socket to your Wi-Fi network

Initial setup must be performed using your home Wi-Fi network.

1.  Ensure your iOS / Android device is connected to your home Wi-Fi

network. Refer to your device manual for further details on connecting

your device to Wi-Fi.

2.   Open the SmartPoint App on your iOS / Android device

3.   Click the settings icon at the bottom of the screen, then select Smart

Setup.

4.

Your wireless network name (SSID) is shown in the smart setup screen

below is an empty field to enter your Wi-Fi network password. Enter

your password into this box.

SSID

5.

Insert the Wifi Socket into the power socket. Turn on the power outlet.

The Wifi Socket will begin to blink red rapidly to indicate it has

entered pairing mode.If the LED does not blink rapidly in red ,

press the button for 5 secs and it should do so.

6.

7.

While the light is flashing rapidly, select the Setup New Socket button in

the SmartPoint App to begin the pairing process.

Once paired, you will see a ‘Config successful’ message. If you do not

see this message repeat step 5 or refer to the FAQ at the back of the

manual for more information.

Note: The configuration process should take about 1 minute or less.

SSID

8.

Once configuration is complete, press Back, then press Devices. Press

the on-screen power button to check that your Wifi Socket is now operating.

9.  Connect the appliance you wish to control to the socket of the Wifi Socket.

SSID

Customise your Wifi Socket

The Wifi Socket can be customised to allow you to easily tell the difference

between multiple plugs connected to your network.

1.   Open the SmartPoint App, and select Customise from the main screen.

2.   Select the Wifi Socket you wish to modify, and you will see the current

and current graphic icon displayed.

3.

4.

5.

To change the name of the Wifi Socket, click the name field,and modify

the text. When you have finished, press done.

Select the graphic you wish to be displayed for this Wifi Socket

by clicking on the icons.

Press Confirm to save your changes, and return to the Customise menu.

Locking your Wifi Socket

You can lock the Wifi Socket to prevent new users from discovering

the Wifi Socket over your local network.

1.   Open the SmartPoint App, and select Customise from the main screen.

2.   Select the Wifi Socket you wish to modify, and you will

see the current name and current graphic icon displayed.

3.   Press the lock icon to lock the Wifi Socket from being

discovered by new users.

Changing your password

Once you have set up your Wifi Socket, you can add a password

to prevent other users accessing the device remotely.

1.   Open the SmartPoint App, and select Customise from the main screen.

2.   Select the Wifi Socket you wish to modify, and you will

see the current name and current graphic icon displayed.

3.   Select Modify Remote Password to set a password for the Wifi socket.

4.   Enter the old password, and your new password, then press OK.

Note: The default password is 888888.

5.   Press Confirm to save your changes, and return to the Customise menu.

Product Operation

Switching On and Off

To control your wifi socket simply follow the below instructions.

1.  Open the SmartPoint App, and select Devices from the main screen.

2.  Select the device you wish to control, then press the power button to

toggle between on and off state.

Tip: The indicator light on the front of your Wifi Socket will be solid

Red when in OFF status, and solid blue when in ON status

Check the Current Status of Your Appliance

1.   Open the SmartPoint App, and select Devices from the main screen.

2.   Select the device you wish to control, refer below.

Offine

On/Off/Offine status               (unreachable)

for selected device                status shown with

broken chain

On status shown with

coloured icon

Off status shown with

grey icon

All Wifi Sockets which have been

programmed with this iOS / Android phone are shown at the

bottom of the screen

Timers and Schedules

Your wifi socket can be used to manage your device power

using automatic timers and schedules.

Setting the Automatic Timer

1.   Connect your iOS/Android Phone to the Wi-Fi network.

2.   Open the SmartPoint App, and select Rules from the main screen.

3.   Select Timing from the menu at the top of the screen, then select the

rule you wish to modify or create a new rule by pressing Add.

4.   Repeat – Select the days you wish your rule to activate. For a single

time only rule, you should select single from the list.

5.

6.

7.

Time – Select the time you wish the action to be performed.

Devices – Select the on/off status you wish for each of your connected

wifi sockets at this time.

Once you have finished making changes, click Save to return to the

Rules menu.

Tip: To return to the Rules menu without saving the timer,

select Back

Using the Countdown Timer

1.   Connect your mobile to a Wi-Fi or 3G/4G network.

2.   Open the SmartPoint App, and select Rules from the main screen.

3.   Select Countdown from the menu at the top of the screen, then select

the wifi socket you wish to control.

4.   You have the option to select a preset time period (A), or create your

own time period (B) using the hour : min : sec selector.

5.   Choose if you would like the wifi socket to be in on status at the

end of the countdown, select “ON”. Select “OFF” if you wish

the wifi socket to be in off status at the end of the countdown.

Note: The timer can support 1 countdown task and 10 timers

A

B

6.

7.

Set the triggering condition:

a. After creation – The countdown timer begins as soon as you press

save, and will operate once only.

b. Once opened – The countdown timer begins every time the wifi

socket is operated. For example, if you switch on, the countdown

timer will switch off after the time period has passed,

i.e. setting a maximum on time for the wifi socket.

Once you are finished making changes, select Save to return to the

Rules menu

Tip: To exit the countdown timer menu without saving your

changes, press Back.

Controlling your wifi socket from a Second iOS / Android Phone

Once you have set up a wifi socket a second iOS/Android phone

can discover it over your home Wi-Fi network and also control it remotely.

- Ensure your iOS / Android phone is connected to your home Wi-Fi network.

- Open the SmartPoint App on your iOS / Android phone.

- Click the Settings icon at the bottom of the screen, then select Smart Setup.

- Your wireless network name (SSID) is shown in the smart setup screen

below is an empty field to enter your Wi-Fi network password. Enter

your password into this box, then press Done.

SSID

5.

Select the Search socket button on your iOS / Android phone.

The App will begin to search for the wifi socket currently connected to

your home Wi-Fi network.

SSID

6.

Once searching is complete, press Back, then click Devices. You should

now see all the wifi sockets connected to your Wi-Fi network.

Note: If you have set the wifi socket to locked mode it will not be

discoverable by other devices. Refer to page 14 for further information.

SSID

Deleting a Device from the SmartPoint App

1.   Open the SmartPoint App on your iOS / Android Phone.

2.   Click the Customise icon at the bottom of the screen, then select the

wifi socket you wish to delete.

3.   Select Delete from the top right corner of the screen. Press OK to

confirm you wish to delete the wifi socket.

4.   You will be returned to the Customise screen, when you can select

another wifi socket if you wish.

Frequently Asked Questions

| Question | Answer |
| --- | --- |
| How can you tell if the Wifi Socket Is successfully paired with my router? | If you are successfully paired the light on the Wifi socket will stay lit. |
| What if I change or update my router? | You will need to reconfigure your Wifi socket to work with your new router, by following the steps on page 6. |
| From what distance can I control the Wifi Socket? | You can control your Wi-Fi Socket from any location in the world, as long as your iOS / Android device can access the internet through Wi-Fi or 4G/3G network. |
| Why has my Wifi Socket turned off unexpectedly? | Your Wi-Fi network may have turned off. Please check and restart your wireless router, then power-on the Wifi socket. |
| What appliances are suitable to use with the Wifi Socket? | The Wifi socket will work with any home appliance that currently plugs into a Smart point and draws under 2400W power. The Wifi socket is best suited to appliances which do not contain internal on/off switching, or which can be switched to an always on state. For example, a traditional lamp would be well suited to the Wifi socket but a touch-lamp which resets to the off position when the power is cut, would not be a good choice for the Wifi socket. |

| Question | Answer |
| --- | --- |
| Can I control multiple Wifi Socket? | Yes, you can add up to 100 Wifi socket to your home network. Note: The number of devices that can be connected to your home network depends on your router / modem. Refer to manufacturer instructions for more information. |
| Can others within my home network control my Wifi Socket? | Yes, see page 21, “Controlling your Wi-Fi Smart Point Plug from a second iOS / Android Phone” in this product manual. Each Wifi Socket also has a lock function which will prevent others on your network from seeing it. This can be controlled from the Settings menu. |
| What can I use the countdown function for? | This will leave appliances on for a time that you set. It can be used as a timer to turn off appliances you only want on for a period of time. For example, set your phone charger to tur n off after 3 hours to stop power draw after your phone has charged. |
| Why is light on the front of my Wifi Socket flashing red? | If the light is slowly flashing red, the device cannot connect to your network. See steps on page 6. If the light is flashing quickly, the unit is in pairing mode. See steps on page 8 to setup network connections. |
| How do I reset the Wifi Socket? | Connect the Wifi Socket to a power source and hold down the power button for 5 seconds to reset the unit. |